GTG WS 2 Hybrid Synthesizer.

A synth for the tweakers.

Phase Modulation Oscillators.

The Modulator ( Osc 2 ) can be sent directly to the filter by means of the Osc 2 Mix knob.

The Modulated signal is sent to the Waveshaper.

Modulation Envelope:

This controls the Phase Modulation. The Joystick Controller sets the amount of modulation ( x )

and the filter frequency ( Y ).

Note: When the Sustain level is set to zero the Modulation Control ( X ) will only have

effect during the Decay time. So keep the Sustain level up if you want to modulate using the

Joystick Controller.

Tempo Sync LFO:

This controls the modulation of the filter as well as the Phase Modulation.

Again this LFO will only have effect on the Phase Modulation during the Decay time,unless the Sustain level is above zero.

The speed of the LFO is set with the Division control.

MIDI control:

Chorus Depth       93

Cut off                74

Resonance           71

Joystick X            91

Joystick Y            92

Vibrato depth      Mod wheel ( 1 )

www.gtgsynths.com